[image: image1.png]DEFY XTREME

CHRONOGRAPH
& i

SFA




Caliber

El Primero 4000 SX
Automatic Chronograph movement



13 1/4``` caliber. Diameter 30 mm. Height : 6.50 mm. 248 components

Balance, chronograph & pallet bridges in shock-absorbing Zenithium Z+

Incabloc® anti-shock device
36’000 vibrations per hour

Power reserve over 50 hours. 31 jewels

Measures short time intervals to a 10th of a second

Automatic winding in both directions

Central rotor on ball bearings

Heavy Metal oscillating weight 

Functions
Hours and minutes in the center



Small seconds counter at 9 o’clock

Date at 4 ½ o’clock
Chronograph:
· Central seconds-hand 

· 30-minute counter at 3 o’clock

· 12-hour counter at 6 o’clock

Case

Black Titanium case. Diameter: 46.5 mm



Black Titanium screw-in push-buttons with carbon fiber guilloché patterns

Black Titanium screw-in crown with specific protecting device  

Graduated unidirectional rotating bezel in black Titanium, fitted with carbon fiber inserts 

Helium valve at 10 o’clock

Black Titanium screw-in case back 
Water-resistance up to 1’000 meters

Dial

Multi-layered see-through structure composed of shock-resistant transparent Hesalite glass, Carbon fiber layer and varnished dial plate
Crystal

Sapphire glass, 3.8 mm thick, with anti-reflective coating on both sides. 
Magnifying lens over the date window at 4 ½ o’clock 
Hands

Black hour, minute, central second hands with White Superluminova inserts


Blued small second, 30-minute & 12-hour chronograph hands with white Superluminova inserts
Bracelet
Black Titanium bracelet with blue-tinted central links. reinforced by composite lateral inserts (which include 20% fiber glass)
Buckle

Brushed black titanium triple folded clasp

EXCLUSIVE LIMITED EDITION: 250 PCS ONLY

